### Section 2-9 Switches

| P 2.9-1 Determine the current, i, at t = 1 s and at t = 4 s for the circuit of Figure P 2.9-1. | Figure P 2.9-1 |
| --- | --- |

#### Solution:

|  |  | At t = 1 s the left switch is open and the right switch is closed so the voltage across the resistor is 10 V. |
| --- | --- | --- |

At t = 4 s the left switch is closed and the right switch is open so the voltage across the resistor is 15 V.

| P 2.9-2 Determine the voltage, v, at t = 1 s and at t = 4 s for the circuit shown in Figure P 2.9-2. | Figure P 2.9-2 |
| --- | --- |

#### Solution:

|  | At t = 1 s the current in the resistor is 3 mA so v = 15 V. At t = 4 s the current in the resistor is 0 A so v = 0 V. |  |
| --- | --- | --- |

Figure P 2.9-3

P 2.9-3 Ideally an open switch is modeled as an open circuit and a closed switch is modeled as a closed circuit. More realistically, an open switch is modeled as a large resistance and a closed switch is modeled as a small resistance.

Figure P 2.9-3a shows a circuit with a switch. In figure P 2.9-3b the switch has been replaced with a resistance. In figure P 2.9-3b the voltage v is given by

Determine the value of v for each of the following cases.

(a)	The switch is closed and Rs = 0 (a short circuit).

(b)	The switch is closed and Rs = 5 Ω.

(c)	The switch is open and Rs = ∞ (an open circuit).

(d)	The switch is open and Rs = 10 kΩ.

#### Solution:

- v = 12 V

- v = 0 V