Use of the Hemocytometer for Determining Total Cell Number in a Liquid Suspension

A method faster than the plate count (see exercise 8) for determining the total number of cells present in a liquid suspension is one in which the hemocytometer is used in conjunction with the microscope. With this method, an aliquot of suspended cells is introduced between a cover glass suspended on mounts above the hemocytometer counting chamber (figure 1). The liquid depth between the cover glass and the counting chamber is 0.1 millimeter (mm).

The counting chamber is divided into a series of small squares in which the smallest squares are 1/400 of a square mm (see the central large square of figure 2). Thus a square mm would contain 400 small squares. The central large square is surrounded by double lines in order to make it easier to visualize when counting cells.

The hemocytometer is difficult to use with small cells because it is relatively thick. It can only be used with the low- and high-power objective lenses, thereby making it difficult to distinguish individual small cells. For white blood cells, yeasts, and larger bacterial cells it is sometimes quite useful. It is used routinely for white blood cell counts and often for following the course of cell growth and multiplication in a liquid medium. For learning purposes yeast is an excellent test organism.

1.	Dilute a test tube suspension of yeast until clouding is barely visible with the naked eye. You may need to further dilute the sample if you find the individual cells too dense to count with the hemocytometer.

2.	Wash the hemocytometer and hemocytometer cover glass with soapy water, rinse with distilled water, and dry the cover glass and hemocytometer counting surface with lens paper or Kimwipes. Make certain that all oily residues are removed from these areas.

3.	Place the cover glass over the counting chamber area.

4.	Tighten the test tube cap of the yeast suspension and shake thoroughly.

5.	Using a Pasteur pipet or plastic dropper, remove approximately 0.3 ml. While controlling the flow with your forefinger, place the tip in the V-shaped indention of the counting chamber adjacent to the edge of the cover glass (figure 1a).

6.	Slowly let the counting chamber fill by capillarity, making sure that the suspension does not go between the cover glass and cover glass mounting supports of the counting chamber (figure 1b). Such an error will raise the height of the fluid under the cover glass which needs to be exactly 0.1 mm. If such an event occurs, return to step 2.

7.	With care, place the hemocytometer on the stage of the microscope such that the hemocytometer counting chamber is centered underneath the low-power objective lens.

8.	Focus and make a total count of the number of cells in a predetermined number of small squares of the double-lined central area of the hemocytometer, for example, 100 small squares. For ease in counting, a budding yeast cell should be treated as one cell. Assuming you find 500 cells in 100 small squares, what is the total number of cells per ml of sample?

9.	Calculations:

a.	100 small squares=1/4 of a square mm; in which instance, 500 cells×;4=2,000 cells per square mm.

b.	As previously mentioned the liquid depth of the counting chamber is 0.1 mm. Thus in order to determine the number of cells per cubic mm of fluid, it becomes necessary next to multiply by a factor of 10 in order to obtain the number of cells per cubic mm. Thus 2,000 cells/sq mm×;10=20,000 cells per cubic mm.

c.	Finally in order to convert a cubic millimeter (mm) to a cubic milliliter (ml), it becomes necessary to multiply by a factor of 1,000 because 1,000 cu mm=1 cu ml. Thereby 20,000 cells per cu mm×;1,000= 20×;106 yeast cells per ml of the original suspension.

Figure 1 (a) Top view of a hemocytometer, showing sample introduction point. (b) Side view of hemocytometer, showing the cover glass resting on the cover glass mounting supports and the (0.1 mm) distance between the top counting surface of the hemocytometer and the underside of the cover glass.

Figure 2 Ruling of a hemocytometer, showing the subdivisions of a central square millimeter. The central square is surrounded by double lines.