Chapter 2 Exercise Solutions

- Examples (a) are:

- Setting switches on modem cards to select IRQ number and COM port

- Placing jumpers on a network card to select IRQ number and I/O address

- Placing jumpers on a network card to select the media that will be attached

- Setting switches on a network card to select IRQ number, I/O address and COM port

- Using a cable tester to located broken cables

- Creating and maintaining and inventory of cables types and cable runs

- Installing software from CD-ROM or diskette

(a) Fortunately, recent network cards do not require jumper placement or switch setting

- Examples are:

- Installation of a network card driver

- Software configuration of a network card's IRQ number, I/O address and COM port

- Selection and configuration of a protocol stack

- Configuring network address selection (Permanent or by DHCP)

- Configuring the subnet mask

- Specifying the network address of the DNS server

- Specifying the network address of the WINS server

- Specifying the network address of the default Gateway

- Selection and configuration of network services such as SNMP agent

- Installation and configuration of dial-up software

- Configuration of routers:

- network address and subnet mask for each port

- transport protocols accepted

- routing protocols such as RIP or OSPF

- routing table for static routing

- configuration password

- configuration of SNMP agent

- Configuration of switches:

- network address and subnet mask

- default gateway address

- virtual private network (VPN) partitions

- DNS server address

- SNMP Read, Write and Trap community strings

- Port configuration such as "enabled" or "disabled."

- Mapping of ports to Ethernet addresses; static, dynamic or restricted

- The items of this list will depend on what software is installed on the device. Examples are:

- PCs

- Memory allocated

- Kernel usage

- Network usage

- Resources accessed by other users on the network

- File backup

- Received/transmitted packet monitoring

- Discovery of network resources

- NMS polling of network devices

- Graphical presentation of network activity based on polling

- Switches and Routers

- Packet filtering based on:

- Ethernet address

- Protocol

- Network address

- Community String

- Authentication

- User ID

- Password

- Probes

- Packet capture

- Packet statistics by:

- Source and/or Destination Ethernet address(es)

- Source and/or Destination network address(es)

- Protocol

- Application

- Examples are:

- Configuration of :

- Desktop Management Initiative (DMI) supported objects on PCs using RPC or SNMP set commands

- Router, Switch and Hub objects using an embedded Telnet server and a Telnet client and password.

- mib-2 objects on PCs, routers, switches and probes using SNMP set commands

- RMON packet filters on probes

- Monitoring of:

- DMI objects on PCs

- mib-2 and RMON objects on routers, switches and hubs

- RMON objects on probes

- Control of:

- Probe Monitoring start

- Probe Monitoring stop

- Examples are:

- Alarm-based events detected by devices such as probes. For example, an alarm could be set to send a trap if the number of packets received by a router during a given interval exceeds a threshold.

- Traps sent by remote devices to management stations

- Denial-of-Access GetResponse message sent by a device to a management station if a GetRequest packet does not contain the correct password (SNMPv3)

- Information in GetResponse packet Error Status fields about VarBind values that are not available (SNMPv2 and SNMPv3)