Lab 1  Office LAN Lab   				          Name:

Objective:   	In this laboratory exercise, you will begin wiring your own computer network.  To begin the exercise, you will need to complete the following:

- Assemble your network team.  Record the name of all team members on this sheet.

- Determine the IP address for each computer in your network.  Examples of IP addresses that can be used are provided in Table 1.  Your instructor will assign you to a LAN and the accompanying IP address

Table 1 - Suggested IP Addresses

| LAN 1 | LAN 2 | LAN 3 | LAN 4 |
| --- | --- | --- | --- |
| 102.123.83.10 | 128.123.83.20 | 122.124.16.30 | 115.91.4.40 |
| 102.123.83.11 | 128.123.83.21 | 122.124.16.31 | 115.91.4.41 |
| 102.123.83.12 | 128.123.83.22 | 122.124.16.32 | 115.91.4.42 |
| 102.123.83.13 | 128.123.83.23 | 122.124.16.33 | 115.91.4.43 |
| 102.123.83.14 | 128.123.83. 24 | 122.124.16.34 | 115.91.4.44 |
| 102.123.83.15 | 128.123.83.25 | 122.124.16.35 | 115.91.4.45 |

All computers should be connected so that each computer in each LAN can communicate with the other computers within their LAN.

Network Equipment Required

- UTP Patch Cables

- Switch / Hub

- Host Computers

- WireShark Protocol Analyzer software

Required Tasks

- document devices connected to the office LAN

- interconnect hosts / make connections from host computer to switch/hub

- verify link lights when making the network connections

- use the ipconfig/all command to view the MAC address

- Configure the Host IP Address

- verify connectivity / ping hosts

Procedure

- Prepare a drawing similar to Fig. 1 based on your lab setup.  You are to create the Office LAN drawing with Microsoft Visio.  You will do this outside of Lab and turn it in next week in lab.  Your network must contain at least 3 computers.  For students who don’t have Microsoft Visio, you can use PowerPoint or any other drawing tool to create your network documentation.

Assign MAC and IP addresses to each computer.

Lab #1 – Office LAN							                              page 2/3

Figure 1  The office LAN topology for this laboratory exercise.

- Configure the IP address for each computer in your LAN.  You will need to follow the steps provided in your book to configure the IP address for your computer.

- Connect all of the computers to the switch.  Verify that you have a link light when making the connection.  Record the port number on the switch so that this information can be indicated on your drawing.

- Once you have made all of the connections, use the ping command to verify that you have network connectivity with all of the other computers on your network.  This requires that you know the IP addresses for each computer on your network.  If you do not have connectivity then troubleshoot and correct the problem.  Record the findings from using the ping command to verify network connectivity.

Lab #1 – Office LAN							                              page 3/3

Assignment - Required

- Submit your office LAN network drawing to the instructor on the date specified. Your drawing must show the host computers, the MAC addresses, the IP addresses and the network topology (e.g. star)

Submit your drawing and memo to the instructor.

The Ping Utility

Usage: ping [-t] [-a] [-n count] [-l size] [-f] [-i TTL] [-v TOS]

Options

-t             	Ping the specifed host until stopped, to stop - type Control-C.

-a             	Resolve addresses to hostnames.

-n count   	Number of echo requests to send.

-l size      	Send buffer size.

-f             	Set Don't Fragment flag in packet.

-i TTL     	Time To Live.

-v TOS    	Type Of Service.

-r count    	Record route for count hops.

-s count    	Timestamp for count hops.

-j host-list 	Loose source route along host-list.

-k host-list	Strict source route along host-list.

-w timeout     	Timeout in milliseconds to wait for each reply.