ASE-Style Review Questions

1.	Which of the following is not included in proper gasoline spill cleanup?

A.	Spread absorbent material to stop the flow and soak up the gasoline.

B.	Immediate containment of the spill.

C.	Use a soap and water solution to wash the gasoline into a container.

D.	Be aware that the absorbent does not reduce flammability concerns.

2.	Technician A says the right-to-know laws require employers to train employees regarding hazardous waste materials.

Technician B says the right-to-know laws have no provisions requiring proper labeling of hazardous materials.

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B

3.	All of the following statements concerning hybrid high-voltage system safety are true, EXCEPT:

A.	Disconnect the motor generators prior to turning the ignition off.

B.	Test the insulating gloves for leaks prior to use.

B.	Do not attempt to test or service the system for 5 minutes after the high-voltage service plug is removed.

C.	Turn the power switch to the off position prior to performing a resistance check.

4.	Technician A says electrical fires are extinguished with Class A fire extinguishers.

Technician B says gasoline is extinguished with Class B fire extinguishers.

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B

5.	Technician A says material safety data sheets (MSDS) explain employers’ and employees’ responsibilities regarding handling and disposal of hazardous materials.

Technician B says material safety data sheets (MSDS) contain specific information about hazardous materials.

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B

6.	Technician A says a solid that ignites spontaneously is considered a hazardous material.

Technician B says a liquid is considered a hazardous material if the vapors on the surface will ignite when exposed to an open flame whose temperature is below 140°F (60°C).

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B

7.	Technician A says safety glasses are sufficient when working with battery acid and refrigerants.

Technician B says full-face shields are designed to provide protection for the face, neck, and eyes of the technician.

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B

8.	Which of the following statements on HEV safety is correct?

- Always place the high-voltage service plug where someone will not accidentally reinstall it.

- Use a voltmeter set on 400 VDC to determine if the high-voltage system voltage is at 0 volts before servicing.

- Test the integrity of the insulating gloves prior to use.

- All of the above.

9.	Which of the following statements concerning gasoline storage is true?

- Always use approved gasoline containers that are painted blue for proper identification.

- Prevent air from entering the container by filling gasoline containers completely full.

- Always transport gasoline containers on plastic bed liners.

- Do not store a partially filled gasoline container for long periods of time.

10.	Technician A says once the service plug is disconnected there is no highvoltage in the vehicle systems.

Technician B says prior to disconnecting the high-voltage service plug, the vehicle must be turned off and the auxiliary battery may need to be disconnected.

Who is correct?

- A only

- B only

- Both A and B

- Neither A nor B