- REVIEW for the Mid-Term Exam

- Commercial Refrigeration

Review of Subjects:

- Know why walk-ins are designed for slightly lower temperatures than reach-ins.

- Know the approximate design box temperatures for:

  - Walk-in refrigerator ______, and walk-in freezer ______.

  - Reach-in refrigerator _____, and reach-in freezer _____.

- Assume a walk-in refrigerator condensing unit and an A/C condensing unit are installed next to each other. They both use R22 refrigerant. Compare their condensing temperatures and their evaporator temperatures and understand why.

- Understand the transfer of heat: That heat goes to cold and that the greater the temperature difference the faster the transfer of heat.

- Know condenser splits of standard medium temperature refrigeration units and the C/S for freezers.

- Given the desired condenser split be able to determine the condensing temperature for different ambient conditions.

- Know what condensing temperature is and how to determine it given the head pressure and the refrigerant type.

- Understand the relationship between ambient temperature, condensing temperature, head pressure, condenser split, and subcooling. Be able to calculate one given the other.

- Know what evaporator TD is and how it is determined.

- What is the standard TD for walk-ins? What is the standard TD for reach-ins? Understand the relationship between box temperature, evaporator temperature, suction pressure, and superheat. Be able to calculate one given the other.

- What is the relationship of moisture removal of an evaporator and its TD?

- What is the standard superheat and subcooling of commercial refrigeration units per T.R.O.T.?

- What is the minimum allowable superheat and subcooling for most refrigeration units? What is the maximum?

- Above what superheat would indicate a starving evaporator?

- Below what superheat would indicate a flooding evaporator?

- Below what subcooling would indicate an undercharged unit?

- Above what subcooling would indicate an overcharged unit?

- Be able to calculate superheat/subcooling from given pressures and temperatures. Then be able to tell what that condition means in terms of starving/flooding or an overcharge/undercharge.

- Know how to take superheat and subcooling and how to calculate it given the pressures, temperatures, and line temperatures.

- Know the difference between off-cycle defrost, planned defrost, and planned defrost with the addition of heat to defrost the evaporator. At what temperatures are each used? (Above 36° is off-cycle, 35° is planned defrost, and below 33° is planned defrost with heat added.)

- What is a multi-circuited evaporator and what type of TEV must be used?

- At what minimum condensing temperature is some type of head pressure control needed?

- How do HPR controls, fan cycle controls, and dampers control head pressure?

- What is the normal condensing temperature maintained by water cooled units?

- On a water cooled unit what is the difference between the temperature of the leaving water and the condensing temperature?

- On a water cooled unit does the incoming water enter the condenser where the warm liquid refrigerant is leaving the condenser, or where the hot gas from the compressor is entering the condenser?

- What is clearance volume when associated with a reciprocating compressor?

- What is volumetric efficiency?

- Know how to determine a compressor’s compression ratio. Also, know which raises the compression ratio more: a drop in suction pressure or a rise in head pressure?

- When diagnosing compressor damage what would most likely cause a broken suction valve, discharge valve, connecting rod, crankshaft, or blown head gasket?

- What is the most likely cause of blow-by or evidence of a worn piston on an air cooled compressor?

- What are the most frequent causes of compressor overheating for an air cooled compressor? How about a suction cooled compressor?

- If you tore down the compressor, what are some of the signs of overheating?

- What is the maximum discharge line temperature?

- What is the most likely cause of random wear on a compressor crankshaft?

- What would you suspect on a suction cooled compressor if the crankshaft bearing closest to the suction service valve was worn, but the bearing closest to the oil pump was not worn?

- What are the causes and how do you prevent flooding?

- What are the causes and how do you prevent slugging (oil or refrigerant)?

- What are the causes and how do you prevent flooded starts?

- What would you suspect on a burned out compressor that also had worn crankshaft bearings? Is the problem electrical or mechanical and why?

- Know how a TEV operates and how to adjust a Sporlan TEV.

- What do the numbers & letters mean in the following Sporlan expansion valves:       FV-1GA, FV-1/2C, FJE-1C, FS-1/3Z, FSE-1ZP?

- Know how to size a TEV based on evaporator Btuh rating.

- What are the opening and closing forces on a TEV? If the bulb lost its charge what would happen and how would the evaporator pressures respond?

- When replacing an existing cap tube with a different sized cap tube remember the following guidelines: Smaller I.D. replacement must be shorter than the original and a larger I.D. replacement must be longer than the original cap tube.

- What is the function of a metering device?

- What are the forces that influence the operation of a fixed metering device?

- What are the forces that influence the operation of a TEV?

- If a TEV is starving, what are 3 things that must be checked to correct it?

- If a TEV is flooding, what are 3 things that must be checked to correct it?

- What can cause a starving cap tube? How would you correct the problem?

- Why is superheat important? When will you get the best superheat reading? What affects superheat?

- According to T.R.O.T., where is the best place to mount the TEV bulb on a horizontal suction line?

- What precautions are necessary when mounting a TEV bulb to a vertical suction line?

- How does a TEV react to head pressure? How about a cap tube?

- How does a TEV react to evaporator load? How about a cap tube?

- How does a TEV react to a dirty or iced evaporator? How about a cap tube?

- What should you try first if a TEV is hunting?

52. What should you do if you think a TEV is blocked or shut down internally?

- What is the temperature differential or swing of an air sensing refrigeration tstat?

- What is the benefit of a coil sensing tstat?

- How do you use the suction service valve to check compressor efficiency?

- Why should you be careful about front seating the discharge service valve?

- What does an EPR do?

- What does a CPR do?

- What are the primary concerns when setting the low pressure control cut-in and cut-out?

- How does an oil safety control work? What two pressures does it measure?

- Why are current sensing relays sometimes used with oil safety controls?

- What is the maximum temperature drop across liquid and suction line filter-driers?

- What are the uses and restrictions concerning suction filter-driers?

- Know what the numbers on a Sporlan filter-drier mean. (Examples are: C304, C163S.)

- What is a suction line accumulator used for and where is it located? Should it be used on any system that has a flooding problem? Why or why not?

- When is an oil separator required? Where is it located?

- Should an oil separator be used to correct a problem of oil loss due to it being trapped somewhere in the system because of improper piping?

- What is a receiver used for and where is it located?

- What is a king valve and what is it used for?

- What is a heat exchanger used for and where is it located?

- What is a vibration eliminator and where is it located?

- What do you have to keep in mind when installing a vibration eliminator?

- How would you know the maximum high pressure control setting on an outdoor unit?

- Know and understand the “Technician Rules of Thumb” (T.R.O.T.) in the book. Be aware of their benefits as well as their limitations. Always use the manufacturer’s recommendations whenever possible.

Information about the exam and test taking tips:

- Do not cram for the test. Do your review over several days or a week.

- Do not wait for the day before the exam! Things happen…

- Bring your P/T chart and a non-programmable calculator.

- Use a pencil, it is easier to change answers before highlighting them.

- Keep up a steady pace and try not to get hung up on a tough question.

- If you are not sure of the correct answer choose one, but mark the question so you can return later to re-check it.

- When you finish the exam, go back and highlight just the letter of the answer. This also gives you an opportunity to make sure you answered all of the questions.

- You can mark on the test paper, make calculations, draw pictures, doodle, or whatever you want to do. Just make sure the person scoring your test knows which answer you have chosen to be the correct one.

- The test has 100 questions worth 1 point each. If you finish those and wish to try more, there are a few Extra Credit questions at ¼ point each.

- The test normally runs an hour and a half from 5:00 PM to 6:30 PM. This is enough time to finish the main part of the test (100 questions). Do not rush in order to do the Extra Credit questions. They do little good if you have not taken the time to concentrate on the first 100 questions.

- However, if you want to start early I will allow you to start at 4:30 which gives you 2 hours.

- Try to get there early so you can take about 5 minutes to get your mind in the “test mode.” Don’t cram; just look over a few of your old quizzes, that’s all.

- Just before you start the exam I suggest you take about 30 seconds to close your eyes, breathe deeply, and relax your mind.

- The exam will be graded in class like the quizzes.

- I will enter your exam grades in my computer and determine your mid-term grade to date. I will give you a print-out so you can verify that my records on attendance, quizzes, and homework agree with your records.