REVIEW for the Final Exam

Commercial Refrigeration

General Notes:

- The final exam covers everything read, tested, or discussed this semester.

- Use a pencil, not a pen. Circle just the letter of the answer. Bring a highlighter and your P/T chart. You may use a non-programmable calculator.

- There are 110 questions. Mark the answers by circling the letter of the correct answer. When finished go back and highlight just the letter you circled. If you do not highlight your answers you will be penalized.

- When you are finished you can try the 10 extra credit questions which only count ½ point for each correct answer. They are more difficult and will be time consuming so fully complete the initial 110 questions first.

- The exam will start at 5:00 pm and will stop at 6:30 pm. The 90 minutes is plenty of time to finish the first 110 questions. If you wish, you may start at 4:30 pm which will give you 2 hours for the exam.

- Approximately 80% is from the second half of the semester, Chapter 7 (Troubleshooting) through Chapter 14. The questions from the first half of the semester will be the more important points covered. Look over the review from the mid-term exam.

- Questions and answers may be similar to the mid-term exam and from the quizzes, but they may not be exactly the same. So read the questions and the answers on the final very carefully.

- You should know the answer by the time you finish reading the question. There is only one best, most correct answer. If you don’t know the correct answer the other choices are there to tempt you, which is why they are called “detractors.”

- Do not get hung up on one question. If you are not sure of the answer choose one answer, then put a mark in the margin so you will double check it when you go back to highlight your answers.

- Study during the week before the exam, do not wait until the weekend to do all your review. Cramming for any test is not productive. Make sure you get a good night’s sleep the night before. Allow extra time to get to class so you will not be rushed. Get to class early enough so you can relax for a few minutes and collect your thoughts. You should not need to study in class before the exam. By then you know it or you don’t. Relaxation is better than last minute cramming.

- On the troubleshooting questions, mark each figure or piece of information as “low, high, normal, flooding, starving,” or whatever applies. Establishing the significance of each point makes diagnosis easier than trying to envision the whole system at once.

Review of Subjects:

- Know the normal box temperatures and evaporator TDs of both reach-in and walk-in refrigerators and freezers.

- Know the normal condenser splits of refrigerators and freezers.

- Know the average normal superheat and subcooling according to T.R.O.T.

- Know the range of superheat and subcooling above and below in which you will have a problem. Also, know what the excessive values indicate is happening.

- If given a box temperature you should be able to tell what the evaporator temperature is supposed to be and what the suction line temperature should be.

- Given an ambient you should be able to tell what the condensing temperature is supposed to be and what the liquid line temperature should be.

- Will an ambient of 70° give a slightly lower condenser split than 95° ambient?

- What is the relationship of evaporator TD to box humidity?

- How does evaporator load affect evaporator temps, pressures, and superheat?

- How do ambient temperatures affect condensing temps, pressures, and subcooling?

- How does a dirty condenser affect condensing temps, pressures, and subcooling?

- How does a dirty or iced evaporator affect evap temps, pressures, and superheat?

- How do condensing and evaporator conditions affect TEV and cap tube systems differently?

- What affect does a low condensing pressure have on a system?

- What are 4 methods of head pressure control and how do they keep the head pressure up?

- On water cooled condensers what is the relationship between the leaving water temperature and the condensing temperature?

- How can you tell which problem (slugging, flooding, or flooded start) caused failure of an air-cooled semi-hermetic compressor given the following problems: broken valves or piston, worn cylinder, random bearing wear on the crankshaft?

- How do you prevent these three problems?

- What are the evaporator symptoms for a (1) TEV system and for (2) a cap-tube system for the following problems: TEV bulb not strapped to the suction line, TEV bulb loses its charge, TEV restricted, liquid line or filter-drier restricted, iced or dirty evaporator, evaporator fan motor problems, high condensing temperatures, low condensing temperatures, low refrigerant, and too much refrigerant?

- What is the difference between air-sensing tstats and coil sensing tstats? How do they prevent frost buildup on a medium temperature evaporator?

- What is the function of an oil separator? Why is it used and how does it affect the piping practices for oil return?

- What does an oil failure control measure and at what pressure will it cut-out?

- What is a CPR, EPR, and HPR? How do they operate and what are they used for?

- What is the difference between an accumulator and a receiver?

- How effective is a filter-drier at removing debris, acid, and moisture?

- How do you know when a filter-drier is at its maximum debris holding capacity?

- How do you know when a filter-drier is at its maximum moisture holding capacity?

- Know motors and how voltage is critical to their proper operation.

- Know contactors, how to choose them, voltage drop, and checking them.

- Know the difference between current and potential relays. Where they are used and how to tell if they are bad.

- Know what is good, and what is not so good, about electronic start relays.

- Know the difference in run and start capacitors, how they are wired, how to check them, and how to choose a replacement.

- Know how overloads are wired, what circuit(s) they open, and how to tell if they are bad.

- Know how to determine what is the cause of a compressor’s starting problems given how it acts (or doesn’t act) on start up.

- When do you need to retrofit a unit with a new refrigerant and when are oil changes needed or not?

- How do you hook up a recovery system and what is the push-pull method?

- How do you charge a retrofitted system if it is a cap-tube, and if it is a TEV system?

- How to charge systems with blends and critically charged systems.

- How to evacuate a system, checking connections, and changing pump oil.

- What is the difference between a single evacuation and a triple evacuation?

- Know how to set a low pressure control and a high pressure control from your pocket charts. What is the maximum high pressure control setting for any refrigerant?

- Know the special considerations in supermarket equipment such as subcooling, superheat, case temperature control, parallel racks, oil separators, case air flow, store humidity, equipment evacuation, installation expertise, and the importance of equipment maintenance.

- Know walk-ins. What is the difference between a storage-only and a production walk-in and how is the equipment size determined?

- What is the most important considerations when installing a walk-in?

- Know the T.R.O.T. of approximate horsepower to btuh ratings for medium and low temperature compressors.

- Know how to pick out the condensing unit and evaporator for each of the following given the box size, horsepower, or btuh requirements: a medium temperature walk-in, for a -10° freezer, and for a 0° freezer.

- Be able to use a pipe sizing chart, now the pipe slope, and equivalent feet.

- Know about pipe type, slope, p-traps, and heating of walk-in drains.

- What are some of the causes of medium temperature and freezer evaporator icing and how do you diagnose them?

- Know ice machines. The importance of installation, water pressure, and drains. What does the warranty cover? How to do a capacity check and how to check proper operation of a water-cooled unit.

- Know health department requirements and what inspectors are looking for. What is the difference between codes by the FDA and the local health departments?

- Know about using regular refrigerators and blast chillers for cooling warm product.

- Know what causes gaskets to deteriorate and what problems are caused by gasket and doors that don’t seal properly.

- Know about basic business practices. How many start per year and how few last?

- What are the considerations of managers when business decisions are made?

- What is the productivity of service techs and what is the average profit?

- What is the importance of hiring well? What about family members, how to figure gross and net profit, and why paperwork is important?