# Commercial Refrigeration QUIZ

Chapter 2 – Evaporators

Name: _____________________	 	Number Correct: ________   Scored by: ___________

- The difference between the evaporator temperature and the suction line temperature at the coil outlet is:

a.  superheat.	b.  latent heat.	c.  artificial heat.	d. extended heat.

- A typical evaporator temperature in an air conditioning system with 75° return air is:

a. 25°.			b. 40°. 		c.  55°. 		d. 75°.

- A typical evaporator temperature in a 35° medium temperature walk-in system is:

a. 25°. 		b. 40°. 		c.  55°. 		d. 75°.

- The TD (temperature difference) between evaporator and box temperature of a reach-in is:

a. 35°. 		b. 20°. 		c. 10°. 		d. 5°.

- The greater the evaporator TD the ______ moisture is removed from the air passing over the evaporator.

a. more		b. less

- If an evaporator has less than 5° superheat it is considered to be ____________.

  - starving. 		b. flooding.		c. about normal.

- If an evaporator has more than 20° superheat it is considered to be ____________.

  - starving. 		b. flooding.		c. about normal.

- If an evaporator has distributor tubes coming off the TEV distributor it is a ______________.

- flooded evaporator.

- cap-tube evaporator.

- multiple-circuit evaporator.

- stamped evaporator.

- What is the temperature differential (cut-in to cut-out) of an air sensing thermostat in a medium temperature refrigeration unit?

  - 2° 			b. 5° 			c. 10° 			d. 20°

- A walk-in at 35° box temperature would require what type of defrost?

  - Off cycle

  - Planned defrost – clock only

- Planned defrost with clock and electric heaters