Match the following people with their contribution to the field of Computer Science, where a name can be used more than once.

  - John Atanasoff and Cilfford Berry

  - Augusta Ada Byron

  - Charles Babbage

  - Luigi F. Menabrea

  - John Mauchly and J. Presper Eckert.

  - John von Neumann

____C___   Inventor of the Analytical Engine

____D___   Published a paper describing the Analytical Engine in French

____A___   Inventor of the ABC

____B___   Wrote detailed instructions for numerical computations, thought to be the first computer program.

____F___   proposed a computing model used in most digital computers

____E___   Inventor of the ENIAC

____B___   envisioned the multidisciplinary potential of the Analytical Engine

- Place the following computers in order from oldest to most recent.

_middle    	ABC

_oldest___	Analytical Engine

_newest__	ENIAC

- List the five steps of the problem solving methodology given in this chapter.

      - Problem Statement

      - Input/Output Description

      - Hand Calculations

      - Algorithm Development

      - Testing

- Convert the decimal number -91 to two's complement using 8 bits, showing all work.

2) 91	|

2) 45 | 1

2) 22 | 1

2) 11 | 0

2)   5 | 1

2)   2 | 1

2)_ 1 | 0

2)   0 | 1

0 1 0 1 1 0 1 1	      is  91

1 0 1 0 0 1 0 0 	is the one's complement of 91

0 0 0 0 0 0 0 1	add one

1 0 1 0 0 1 0 1	is the two's complement representation of -91

- Convert the binary number 1010110001010111 to hexadecimal, showing any work.

1 0 1 0|1 1 0 0|0 1 0 1|0 1 1 1

A         C         5         7		10101100010101112 is AC5716

- Which of the following is not computer hardware?

  - a printer

  - the hard disk

  - the operating system

  - the mouse

- Which of the following is not computer software?

  - a USB memory stick

  - the operating system

  - a C++ compiler

  - a spreadsheet

- Which of the following is responsible for interpreting the instructions stored in memory?

  - the ALU (arithmetic logic unit)

  - the control unit

  - the software

  - the accumulator

- High-speed registers for the temporary storage of values are found in …

  - the computers hard disk

  - the computer's primary memory

  - the accumulator

  - the C++ program

- Which of the following is not a function of a utility provided by an operating system?

  - copying files

  - listing files

  - printing files

  - translating C++ programs

- Which of the following is not a part of a GUI (Graphical User Interface)?

  - icons

  - menus

  - a mouse

  - text boxes

- Which of the following is not considered to be application software?

  - database management software

  - spreadsheet programs

  - an operating system

  - mathematical computation software

- Which of the following is not an example of mathematical computation software?

  - Maple

  - MATLAB

  - Mathematica

  - MySQL

- Which of the following languages is unique to a computer design where the instructions are binary strings?

  - assembly language

  - Basic

  - C++

  - machine language

- Semantics are the set of grammar rules for a high-level language.

  - true

  - false

- The CPU refers to the control unit and the ALU.(arithmetic logic unit)

  - true

  - false

- A modern digital computer represents values in memory in base 10.

  - true

  - false

- A compiler is hardware used to translate a C++ program into machine language.

  - true

  - false

- The source file is the machine language version of a C++ program.

  - true

  - false

- Linking is the process of combining the object file with other machine language statements and loading the executable program into memory.

  - true

  - false

- Program bugs are errors which occur during the compilation step for preparing a C++ program for execution.

  - true

  - false

- The word size is the size of the registers in the ALU, and therefore depends on the design of the processor.

  - true

  - false

- A byte is by definition 8 bits.

  - true

  - false

- Every real number can be represented exactly in binary in the computer's memory.

  - true

  - false